# User Manual – Si5351 + ATtiny1614 Frequency Generator

# 1. Introduction

This frequency generator is built around the ATtiny1614 microcontroller and the Si5351 clock generator module, capable of producing three independent outputs (CLK0, CLK1, CLK2) with frequencies ranging from 8 kHz to 150 MHz. The user interface consists of a 16x2 I2C LCD display and a rotary encoder with push button. The device allows frequency adjustment, step selection, output enable/disable, and crystal calibration.

# 2. User Interface

[ATtiny1614 Pinout]

[Rotary Encoder]

[LCD 1602 I2C]

Rotary encoder: navigation and value adjustment

Encoder button: selection and return

16x2 LCD: displays menus and values

# 3. Menu Navigation

Rotate encoder: change option

Press encoder: enter submenu or confirm

"bak": return to previous menu

# 4. Main Menu

At startup, the following options are available: CLK0, CLK1, CLK2, and Calib. Rotate the encoder to select, press to enter.

# 5. CLK Menu

Each output provides:

Freq (frequency adjustment)

OnOff (enable/disable)

Step (step size selection)

bak (return)

# 6. Frequency Editing

Frequency is adjusted by rotating the encoder. Limits: 8 kHz – 160 MHz. Display format: MHz with 6 decimals, kHz with 3 decimals. Frequency is automatically saved to EEPROM.

# 7. Step Selection

Available steps:

1 Hz

10 Hz

100 Hz

1 kHz

10 kHz

100 kHz

1 MHz

# 8. Output Enable/Disable

Allows switching each CLK output between ON and OFF.

# 9. Calibration Menu

Allows adjusting the Si5351 crystal calibration value. Changes apply in real time and are saved automatically.

# 10. EEPROM Storage

Automatically stored:

Frequencies

ON/OFF state

Selected step

Calibration value

# 11. Frequency Display Format

MHz: X.YYYYYYMHz

kHz: XXX.XXXkHz

# 12. Recommendations and Limits

Do not set frequencies below 8 kHz

Do not exceed 160 MHz

Use a high-quality 25 MHz crystal

Recalibrate periodically if drift is observed